Make New Project

Select Board

Click Add Sources -Next – Crete File - Name (oledControl.v) and ok

All Codes attached at the end of Document

Click Add Sources -Next – Crete File - Name (delayGen.v) and ok

Click Add Sources -Next – Crete File - Name (spiControl.v) and ok

Click Add Sources -Next – Crete File - Name (charROM.v) and ok

Click Add Sources -Next – Crete File - Name (top.v) and ok

Click Constraint Add source create constraint File “oledControl.xdc”

Set Package Pins

Now Run Synthesis – Run Implementation – Generate Bitstream

Now Open Hardware Manager – Open Target – Auto Connect – Program device.

TOP.V Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 09/25/2024 11:21:49 AM

// Design Name:

// Module Name: top

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module top(

input  clock, //100MHz onboard clock

input  reset,

//oled interface

output oled_spi_clk,

output oled_spi_data,

output oled_vdd,

output oled_vbat,

output oled_reset_n,

output oled_dc_n

);

localparam myString ="Farhan Siddiqui";

localparam StringLen = 15;

reg [1:0] state;

reg [7:0] sendData;

reg sendDataValid;

integer byteCounter;

wire sendDone;

localparam IDLE = 'd0,

SEND = 'd1,

DONE = 'd2;

always @(posedge clock)

begin

if(reset)

begin

state <= IDLE;

byteCounter <= StringLen;

sendDataValid <= 1'b0;

end

else

begin

case(state)

IDLE:begin

if(!sendDone)

begin

sendData <= myString[(byteCounter*8-1)-:8];

sendDataValid <= 1'b1;

state <= SEND;

end

end

SEND:begin

if(sendDone)

begin

sendDataValid <= 1'b0;

byteCounter <= byteCounter-1;

if(byteCounter != 1)

state <= IDLE;

else

state <= DONE;

end

end

DONE:begin

state <= DONE;

end

endcase

end

end

oledControl OC(

.clock(clock), //100MHz onboard clock

.reset(reset),

//oled interface

.oled_spi_clk(oled_spi_clk),

.oled_spi_data(oled_spi_data),

.oled_vdd(oled_vdd),

.oled_vbat(oled_vbat),

.oled_reset_n(oled_reset_n),

.oled_dc_n(oled_dc_n),

//

.sendData(sendData),

.sendDataValid(sendDataValid),

.sendDone(sendDone)

);

Endmodule

oledControl.V Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 09/25/2024 09:59:42 AM

// Design Name:

// Module Name: oledControl

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module oledControl(

input  clock, //100MHz onboard clock

input  reset,

//oled interface

output wire oled_spi_clk,

output wire oled_spi_data,

output reg oled_vdd,

output reg oled_vbat,

output reg oled_reset_n,

output reg oled_dc_n,

//

input [6:0] sendData,

input       sendDataValid,

output reg  sendDone

);

reg [4:0] state;

reg [4:0] nextState;

reg startDelay;

reg [7:0] spiData;

reg spiLoadData;

wire delayDone;

wire spiDone;

reg [1:0] currPage;

wire [63:0] charBitMap;

reg [7:0] columnAddr;

reg [3:0] byteCounter;

localparam  IDLE  = 'd0,

DELAY = 'd1,

INIT  = 'd2,

RESET = 'd3,

CHRG_PUMP = 'd4,

CHRG_PUMP1 = 'd5,

WAIT_SPI = 'd6,

PRE_CHRG = 'd7,

PRE_CHRG1 = 'd8,

VBAT_ON = 'd9,

CONTRAST = 'd10,

CONTRAST1 = 'd11,

SEG_REMAP = 'd12,

SCAN_DIR = 'd13,

COM_PIN = 'd14,

COM_PIN1 = 'd15,

DISPLAY_ON = 'd16,

FULL_DISPLAY = 'd17,

DONE = 'd18,

PAGE_ADDR = 'd19,

PAGE_ADDR1 = 'd20,

PAGE_ADDR2 = 'd21,

COLUMN_ADDR = 'd22,

SEND_DATA = 'd23;

always @(posedge clock)

begin

if(reset)

begin

state <= IDLE;

nextState <= IDLE;

oled_vdd <= 1'b1;

oled_vbat <= 1'b1;

oled_reset_n <= 1'b1;

oled_dc_n <= 1'b1;

startDelay <= 1'b0;

spiData <= 8'd0;

spiLoadData <= 1'b0;

currPage <= 0;

sendDone <= 0;

columnAddr <= 0;

end

else

begin

case(state)

IDLE:begin

oled_vbat <= 1'b1;

oled_reset_n <= 1'b1;

oled_dc_n <= 1'b0;

oled_vdd <= 1'b0;

state <= DELAY;

nextState <= INIT;

end

DELAY:begin

startDelay <= 1'b1;

if(delayDone)

begin

state <= nextState;

startDelay <= 1'b0;

end

end

INIT:begin

spiData <= 'hAE;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

oled_reset_n <= 1'b0;

state <= DELAY;

nextState <= RESET;

end

end

RESET:begin

oled_reset_n <= 1'b1;

state <= DELAY;

nextState <= CHRG_PUMP;

end

CHRG_PUMP:begin

spiData <= 'h8D;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= CHRG_PUMP1;

end

end

WAIT_SPI:begin

if(!spiDone)

begin

state <= nextState;

end

end

CHRG_PUMP1:begin

spiData <= 'h14;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= PRE_CHRG;

end

end

PRE_CHRG:begin

spiData <= 'hD9;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= PRE_CHRG1;

end

end

PRE_CHRG1:begin

spiData <= 'hF1;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= VBAT_ON;

end

end

VBAT_ON:begin

oled_vbat <= 1'b0;

state <= DELAY;

nextState <= CONTRAST;

end

CONTRAST:begin

spiData <= 'h81;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= CONTRAST1;

end

end

CONTRAST1:begin

spiData <= 'hFF;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= SEG_REMAP;

end

end

SEG_REMAP:begin

spiData <= 'hA0;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= SCAN_DIR;

end

end

SCAN_DIR:begin

spiData <= 'hC0;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= COM_PIN;

end

end

COM_PIN:begin

spiData <= 'hDA;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= COM_PIN1;

end

end

COM_PIN1:begin

spiData <= 'h00;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= DISPLAY_ON;

end

end

DISPLAY_ON:begin

spiData <= 'hAF;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= PAGE_ADDR;//FULL_DISPLAY;

end

end

PAGE_ADDR:begin

spiData <= 'h22;

spiLoadData <= 1'b1;

oled_dc_n <= 1'b0;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= PAGE_ADDR1;

end

end

PAGE_ADDR1:begin

spiData <= currPage;//start page address

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

currPage <= currPage+1;

nextState <= PAGE_ADDR2;

end

end

PAGE_ADDR2:begin

spiData <= currPage;//start page address

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= COLUMN_ADDR;

end

end

COLUMN_ADDR:begin

spiData <= 'h10;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= DONE;

end

end

/*FULL_DISPLAY:begin

spiData <= 'hA5;

spiLoadData <= 1'b1;

if(spiDone)

begin

spiLoadData <= 1'b0;

state <= WAIT_SPI;

nextState <= DONE;

end

end*/

DONE:begin

sendDone <= 1'b0;

if(sendDataValid & columnAddr != 128 & !sendDone)

begin

state <= SEND_DATA;

byteCounter <= 8;

end

else if(sendDataValid & columnAddr == 128 & !sendDone)

begin

state <= PAGE_ADDR;

columnAddr <= 0;

byteCounter <= 8;

end

end

SEND_DATA:begin

spiData <= charBitMap[(byteCounter*8-1)-:8];

spiLoadData <= 1'b1;

oled_dc_n <= 1'b1;

if(spiDone)

begin

columnAddr <= columnAddr + 1;

spiLoadData <= 1'b0;

state <= WAIT_SPI;

if(byteCounter != 1)

begin

byteCounter <= byteCounter - 1;

nextState <= SEND_DATA;

end

else

begin

nextState <= DONE;

sendDone <= 1'b1;

end

end

end

endcase

end

end

delayGen DG(

.clock(clock),

.delayEn(startDelay),

.delayDone(delayDone)

);

spiControl SC(

.clock(clock), //On-board Zynq clock (100 MHz)

.reset(reset),

.data_in(spiData),

.load_data(spiLoadData), //Signal indicates new data for transmission

.done_send(spiDone),//Signal indicates data has been sent over spi interface

.spi_clock(oled_spi_clk),//10MHz max

.spi_data(oled_spi_data)

);

charROM CR(

.addr(sendData),

.data(charBitMap)

);

Endmodule

delayGen Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 09/25/2024 10:48:18 AM

// Design Name:

// Module Name: delayGen

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module delayGen(

input clock,

input delayEn,

output reg delayDone

);

reg [17:0] counter;

always @(posedge clock)

begin

if(delayEn & counter!=200000)

counter <= counter+1;

else

counter <= 0;

end

always @(posedge clock)

begin

if(delayEn & counter==200000)

delayDone <= 1'b1;

else

delayDone <= 1'b0;

end

endmodule

spiControl Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 09/25/2024 10:49:05 AM

// Design Name:

// Module Name: spiControl

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module spiControl(

input  clock, //On-board Zynq clock (100 MHz)

input  reset,

input [7:0] data_in,

input  load_data, //Signal indicates new data for transmission

output reg done_send,//Signal indicates data has been sent over spi interface

output     spi_clock,//10MHz max

output reg spi_data

);

reg [2:0] counter=0;

reg [2:0] dataCount;

reg [7:0] shiftReg;

reg [1:0] state;

reg clock_10;

reg CE;

assign spi_clock = (CE == 1) ? clock_10 : 1'b1;

always @(posedge clock)

begin

if(counter != 4)

counter <= counter + 1;

else

counter <= 0;

end

initial

clock_10 <= 0;

always @(posedge clock)

begin

if(counter == 4)

clock_10 <= ~clock_10;

end

localparam IDLE = 'd0,

SEND = 'd1,

DONE = 'd2;

always @(negedge clock_10)

begin

if(reset)

begin

state <= IDLE;

dataCount <= 0;

done_send <= 1'b0;

CE <= 0;

spi_data <= 1'b1;

end

else

begin

case(state)

IDLE:begin

if(load_data)

begin

shiftReg <= data_in;

state <= SEND;

dataCount <= 0;

end

end

SEND:begin

spi_data <= shiftReg[7];

shiftReg <= {shiftReg[6:0],1'b0};

CE <= 1;

if(dataCount != 7)

dataCount <= dataCount + 1;

else

begin

state <= DONE;

end

end

DONE:begin

CE <= 0;

done_send <= 1'b1;

if(!load_data)

begin

done_send <= 1'b0;

state <= IDLE;

end

end

endcase

end

end

endmodule

charROM Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 09/25/2024 10:42:57 AM

// Design Name:

// Module Name: charROM

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module charROM(

input [6:0] addr,

output [63:0] data

);

reg [63:0] mem [127:0]; //model of memory

initial

begin

mem[0] = 'h0000000000000000;

mem[1] = 'h0000000000000000;

mem[2] = 'h0000000000000000;

mem[3] = 'h0000000000000000;

mem[4] = 'h0000000000000000;

mem[5] = 'h0000000000000000;

mem[6] = 'h0000000000000000;

mem[7] = 'h0000000000000000;

mem[8] = 'h0000000000000000;

mem[9] = 'h0000000000000000;

mem[10] = 'h0000000000000000;

mem[11] = 'h0000000000000000;

mem[12] = 'h0000000000000000;

mem[13] = 'h0000000000000000;

mem[14] = 'h0000000000000000;

mem[15] = 'h0000000000000000;

mem[16] = 'h0000000000000000;

mem[17] = 'h0000000000000000;

mem[18] = 'h0000000000000000;

mem[19] = 'h0000000000000000;

mem[20] = 'h0000000000000000;

mem[21] = 'h0000000000000000;

mem[22] = 'h0000000000000000;

mem[23] = 'h0000000000000000;

mem[24] = 'h0000000000000000;

mem[25] = 'h0000000000000000;

mem[26] = 'h0000000000000000;

mem[27] = 'h0000000000000000;

mem[28] = 'h0000000000000000;

mem[29] = 'h0000000000000000;

mem[30] = 'h0000000000000000;

mem[31] = 'h0000000000000000;

mem[32] = 'h0000000000000000;

mem[33] = 'h0000005f00000000;

mem[34] = 'h0000030003000000;

mem[35] = 'h643c26643c262400;

mem[36] = 'h2649497f49493200;

mem[37] = 'h4225120824522100;

mem[38] = 'h20504e5522582800;

mem[39] = 'h0000000300000000;

mem[40] = 'h00001c2241000000;

mem[41] = 'h00000041221c0000;

mem[42] = 'h0015150e0e151500;

mem[43] = 'h0008083e08080000;

mem[44] = 'h0000005030000000;

mem[45] = 'h0008080808080000;

mem[46] = 'h0000004000000000;

mem[47] = 'h4020100804020100;

mem[48] = 'h003e4141413e0000;

mem[49] = 'h0000417f40000000;

mem[50] = 'h00426151496e0000;

mem[51] = 'h0022414949360000;

mem[52] = 'h001814127f100000;

mem[53] = 'h0027494949710000;

mem[54] = 'h003c4a4948700000;

mem[55] = 'h004321110d030000;

mem[56] = 'h0036494949360000;

mem[57] = 'h00060949291e0000;

mem[58] = 'h0000001200000000;

mem[59] = 'h0000005230000000;

mem[60] = 'h0000081414220000;

mem[61] = 'h0014141414141400;

mem[62] = 'h0000221414080000;

mem[63] = 'h0002015905020000;

mem[64] = 'h3e415d554d512e00;

mem[65] = 'h407c4a094a7c4000;//A

mem[66] = 'h417f494949493600;//B

mem[67] = 'h1c22414141412200;//C

mem[68] = 'h417f414141221c00;

mem[69] = 'h417f49495d416300;

mem[70] = 'h417f49091d010300;

mem[71] = 'h1c224149493a0800;

mem[72] = 'h417f0808087f4100;

mem[73] = 'h0041417F41410000;

mem[74] = 'h304041413F010100;

mem[75] = 'h417f080c12614100;

mem[76] = 'h417f414040406000;

mem[77] = 'h417f420c427f4100;

mem[78] = 'h417f420c117f0100;

mem[79] = 'h1c22414141221c00;

mem[80] = 'h417f490909090600;

mem[81] = 'h0c12212161524c00;

mem[82] = 'h417f090919694600;

mem[83] = 'h6649494949493300;

mem[84] = 'h0301417f41010300;

mem[85] = 'h013f4140413f0100;

mem[86] = 'h010f3140310f0100;

mem[87] = 'h011f6114611f0100;

mem[88] = 'h4141360836414100;

mem[89] = 'h0103447844030100;

mem[90] = 'h4361514945436100;

mem[91] = 'h00007f4141000000;

mem[92] = 'h0102040810204000;

mem[93] = 'h000041417f000000;

mem[94] = 'h0004020101020400;

mem[95] = 'h0040404040404000;

mem[96] = 'h0001020000000000;

mem[97] = 'h00344a4a4a3c4000;

mem[98] = 'h00413f4848483000;

mem[99] = 'h003c424242240000;

mem[100] = 'h00304848493f4000;

mem[101] = 'h003c4a4a4a2c0000;

mem[102] = 'h0000487e49090000;

mem[103] = 'h00264949493f0100;

mem[104] = 'h417f480444784000;

mem[105] = 'h0000447d40000000;

mem[106] = 'h000040443d000000;

mem[107] = 'h417f101824424200;

mem[108] = 'h0040417f40400000;

mem[109] = 'h427e027c027e4000;

mem[110] = 'h427e4402427c4000;

mem[111] = 'h003c4242423c0000;

mem[112] = 'h00417f4909090600;

mem[113] = 'h00060909497f4100;

mem[114] = 'h00427e4402020400;

mem[115] = 'h00644a4a4a360000;

mem[116] = 'h00043f4444200000;

mem[117] = 'h00023e4040227e40;

mem[118] = 'h020e3240320e0200;

mem[119] = 'h021e6218621e0200;

mem[120] = 'h4262140814624200;

mem[121] = 'h0143453805030100;

mem[122] = 'h004662524a466200;

mem[123] = 'h0000083641000000;

mem[124] = 'h0000007f00000000;

mem[125] = 'h0000004136080000;

mem[126] = 'h0018080810101800;

mem[127] = 'hAA55AA55AA55AA55;

end

assign data = mem[addr];

endmodule

OledControl.xdc Code

set_property PACKAGE_PIN Y9 [get_ports clock]

set_property IOSTANDARD LVCMOS18 [get_ports clock]

set_property PACKAGE_PIN U10 [get_ports oled_dc_n]

set_property PACKAGE_PIN U9 [get_ports oled_reset_n]

set_property PACKAGE_PIN AB12 [get_ports oled_spi_clk]

set_property PACKAGE_PIN AA12 [get_ports oled_spi_data]

set_property PACKAGE_PIN U11 [get_ports oled_vbat]

set_property PACKAGE_PIN U12 [get_ports oled_vdd]

set_property PACKAGE_PIN P16 [get_ports reset]

set_property IOSTANDARD LVCMOS18 [get_ports oled_dc_n]

set_property IOSTANDARD LVCMOS18 [get_ports oled_reset_n]

set_property IOSTANDARD LVCMOS18 [get_ports oled_spi_clk]

set_property IOSTANDARD LVCMOS18 [get_ports oled_spi_data]

set_property IOSTANDARD LVCMOS18 [get_ports oled_vbat]

set_property IOSTANDARD LVCMOS18 [get_ports oled_vdd]

set_property IOSTANDARD LVCMOS18 [get_ports reset]

create_clock -period 10.000 -name clock -waveform {0.000 5.000} clock